Leak Detection and Repair Log

Perchloroethylene Dry Cleaners

Under NESHAP M, §63.322, inspections for perceptible leaks of perchloroethylene must be performed (while the dry cleaning system is operating) weekly for large area sources and major sources, and bi-weekly for small area sources.  The following components must be inspected: hose and pipe connections, fittings, couplings and valves; door gaskets and seatings; filter gaskets and seatings; pumps; solvent tanks and containers; water separators; muck cookers; stills; exhaust dampers; diverter valves; and all filter housings.  Perceptible leaks are those that are obvious from: (1) odor; (2) visual observation, such as pools or droplets of liquid; or (3) detection of gas flow by passing the fingers over the surface of equipment.  Additionally, both small and large area sources must conduct a monthly inspection in accordance with §63.322(o)(1)(i) using a halogenated hydrocarbon detector or a PCE gas analyzer operated according to manufacturer’s instructions.  Major sources must conduct the inspections using a PCE gas analyzer operated according to EPA Method 21.  Each inspection done with a detector satisfies one inspection for perceptible leaks.

Any leaks detected during an inspection must be repaired within 24 hours.  Any needed repair parts must be ordered within 2 working days of detecting a leak, and must be installed within 5 working days after receipt.

| Date | Inspector’s Name | Indicate “Perceptible” or “Detector” Inspection | Leaky component name or location (or write “none”) | Date part ordered | Date part received | Repair date |
| --- | --- | --- | --- | --- | --- | --- |